AI算力基础设施驱动下城市人工水系统
代谢响应与条件化承载边界研究

博士学位论文开题报告（1.1精修版）

研究方向：城市水系统与资源环境管理　　拟采用方法：综合建模、情景分析与不确定性分析

摘　要

生成式人工智能正在把数据中心推向高功率密度、长时间连续运行和快速空间集聚的新阶段。大模型训练、推理、视频生成和自主智能体等任务不断提高服务器负载，计算活动由数字空间转化为持续的散热、冷却和基础设施需求。已有研究已经估算了数据中心的电力、水足迹和碳排放，也比较了空气冷却、液冷、蒸发冷却和再生水等技术路线。现有成果主要回答“一个模型、一个服务器或一个数据中心消耗多少水”，城市规划还需要进一步回答：当AI算力以持续增长的工业负荷进入城市以后，它如何改变原水取用、给水处理、用户用水、污水回流、再生水生产及最终排放之间的关系？

本研究以城市人工水系统为研究对象，提出“设施过程—城市代谢—条件化承载边界”的统一研究框架。研究首先把AI算力中心表示为城市水系统中的特殊节点，沿“IT容量—实际负载—能耗—热排散—冷却补水—蒸发与排污—污水处理—再生水回用”建立水量和水质的闭合核算；其次比较AI负荷与等年用水量普通工业冷却负荷的差异，分析AI的连续性、气候敏感性和峰值同步如何影响城市水代谢；最后以条件化AI水系统承载容量（CAWCC）表示在给定城市状态、技术架构、水源配置、气候压力和服务优先级下可支持的最大AI IT容量，识别容量边界及其主导约束。

论文拟回答三个科学问题：其一，AI负荷在什么条件下由单纯水量增加转变为水源结构和消耗—回流关系的重构；其二，年均、日尺度和极端高温/干旱条件下的容量边界为何不同，哪些变量决定“平均安全而峰值失稳”；其三，AI扩张或再生水、冷却和设施扩容干预如何引起水源、给水、污水、再生水和局部接入瓶颈的迁移。研究将采用深度案例城市、外部锚点城市和连续参数空间的分层证据设计，以过程验证、独立时间验证、外部核验和结构对照共同约束结论。预期形成关于AI城市水代谢重构、时间尺度依赖承载边界和瓶颈转换机制的科学认识，并为算力布局与城市水务协同规划提供可解释的容量依据。

关键词：人工智能；数据中心；城市人工水系统；城市代谢；再生水；承载容量；峰值压力；瓶颈转换

第1章　背景与意义

1.1　AI发展正在由算法竞争转向基础设施竞争

人工智能产业的竞争焦点已经从单一算法性能扩展到模型、芯片、算力集群和数据中心基础设施的协同能力。大语言模型和多模态模型的参数规模、上下文长度以及推理步数不断增加，视频生成、科学计算和自主智能体又将计算从一次性训练推向高频、连续的在线服务。计算需求的增长具有明确的物理后果：服务器功率密度提高，机房热流密度上升，冷却系统从辅助设备变成决定数据中心能耗和水耗的关键环节。

在传统数字基础设施中，计算活动常被视为“无形服务”，城市规划因而容易把数据中心看作普通建筑或一般工业用户。但AI数据中心具有高连续负荷、设备集中、对供电和散热可靠性要求高等特征，其水需求并非只取决于年均用水量，而同时受湿球温度、冷却架构、负载率、浓缩倍数、再生水水质和局部管网能力影响。AI基础设施因此成为连接数字经济与城市物质系统的新型工业节点。

国际能源署、美国劳伦斯伯克利国家实验室等机构的情景研究表明，数据中心用电增长将成为未来电力系统的重要新增负荷，AI服务器是其中的关键驱动。电力增长会以余热和冷却需求的形式反馈到水系统。若仍以单设施年度WUE作为唯一指标，就会忽略水源、污水处理和再生水系统在空间和时间上的约束，也无法解释相同IT容量在不同城市产生的不同风险。

1.2　AI水资源影响的研究对象需要从设施足迹转向城市系统

现有数据中心水研究至少包含两个边界：一是冷却塔、蒸发器和辅助设备在所在地直接发生的取水与耗水；二是发电和燃料供应在异地形成的间接水足迹。两者在空间位置、治理主体和可干预手段上并不相同。把二者简单相加，虽然可以得到一个综合足迹，却会掩盖城市本地供水系统真正承受的压力；只计算直接用水，又会低估能源结构变化带来的环境后果。

设施水足迹还存在“同量异压”现象。两个数据中心即使具有相同的年度补水量，如果一个使用本地淡水、在夏季高温时段集中取水，另一个主要使用稳定的再生水并通过储水削峰，它们对城市的影响将完全不同。前者可能首先触发水源或给水峰值约束，后者则可能把压力转移到污水深度处理、输配或水质控制环节。由此可见，城市规划需要的是水流路径和服务可靠性的解释，而不仅是一个年度总量。

城市人工水系统是一个开放而具有循环性的网络。外部原水经给水处理后进入居民、商业、工业和公共设施；一部分水以蒸发、产品含水或其他形式离开可管理系统，另一部分形成污水进入收集和处理环节；处理出水既可以排放，也可以经深度处理转化为再生水，重新供给工业、生态或冷却用途。AI节点进入以后，既会增加冷却蒸发消耗，也可能通过排污增加污水和再生水的潜在来源，因此其影响可能表现为网络结构重组。

1.3　研究的理论意义

拓展城市代谢理论：把“计算需求驱动的物理资源流”作为城市代谢的新型节点，连接数字基础设施研究与城市水循环研究。

发展综合水系统模拟：在供水—用水—污水—再生水框架中显式区分Withdrawal、Consumption与Return Flow，并将水量、水质、储存和峰值服务纳入统一边界。

深化承载力理论：把承载对象从人口或经济规模转化为可度量的AI IT容量，讨论资源量、基础设施能力和风险容忍共同形成的条件边界。

1.4　研究的实践意义

为地方政府提供算力项目与水资源、水务设施协同布局的分析框架，把年均资源量、峰值服务和基础设施余量纳入重大项目论证。

为数据中心规划者比较空气/液冷、干式/湿式排热、淡水/再生水配置的系统影响，识别节水措施可能造成的能耗或碳排放转移。

为水务部门评估AI扩张对给水、污水、再生水处理和局部接入能力的增量压力，并判断扩容措施的有效区间。

1.5　研究对象、边界与核心概念

本研究的核心对象是“AI算力基础设施进入城市人工水系统后的系统响应”，研究重点放在城市尺度耦合及其科学机制。研究边界包括城市外部水源、原水和给水处理、配水、终端用户、AI数据中心、污水收集与处理、再生水处理与输配以及最终排放；雨洪、地下水和完整电网仅在具有清晰输入、输出和储量关系的扩展情景中讨论。CAWCC不是脱离条件的固定常数，而是在明确城市状态、技术架构、水源政策、气候情景、基本服务约束和风险容忍水平下的最大可行AI IT容量。

1.6　候选案例城市与数据落点

本研究先提出候选城市，再依据数据审计确定深度案例。北京、天津和贵阳构成当前具有互补性的候选集合：北京具有连续发布的水资源公报、水务统计年鉴和较完整的再生水利用资料，适合刻画超大城市的资源紧约束与再生水系统；天津的水资源公报同时报告地表水、地下水、非常规水源、生活与工业用水，适合构造水源结构和季节压力对照；贵阳所在贵州省的水资源公报提供行政分区水资源量与人均水资源量，适合形成相对水资源充裕、气候条件不同的外部锚点[85–87]。候选城市用于构造可比较的状态空间：深度案例承担日尺度过程校准，其他城市承担参数范围、响应方向和瓶颈类型核验。

公开资料显示，候选城市之间具有可用于机制比较的状态差异：北京市水务资料连续发布年度水资源公报和水务统计年鉴，2023年再生水利用量达到12.77亿立方米，规模以上再生水厂81座、管线长度2274公里；天津市2024年水资源公报报告供水总量32.0017亿立方米，其中非常规水源供水量6.7304亿立方米；贵州省2024年水资源公报给出贵阳行政分区水资源总量39.59亿立方米、人均水资源量572立方米。这些公开口径能够直接支撑水源结构、再生水供给和城市状态变量的初步构建，再结合逐日气象与设施资料完成深度案例筛选[85–89]。

第2章　文献综述与研究空白

2.1　城市代谢与综合城市水系统研究

Wolman提出城市代谢概念后，研究逐步由城市资源输入和输出清单发展到物质流网络、生态网络和规划决策。早期研究强调城市是依赖外部资源输入并产生废弃物输出的开放系统；后续研究进一步关注资源在城市内部的转换、储存、循环和反馈。对于水系统而言，代谢视角的价值在于把供水、用水、污水、再生利用和环境排放放到同一张网络中，从而识别水量之外的路径结构。

综合城市水系统模型已经形成可供本研究继承的理论和工具基础。WaterMet²把供水、需求、污水和环境绩效置于同一评价框架，WSIMOD则以节点和弧线连接水量与污染物通量，强调可组合、可扩展和守恒检查[15–19]。这些研究说明综合模型能够支撑长期规划与方案比较；同时，模型中的用户节点仍主要按居民、商业和传统工业划分，AI数据中心这种高功率密度、连续运行且对气候敏感的负荷尚未成为城市水循环中的独立研究对象。

城市代谢研究的另一个不足是，部分研究以年度物质流清单为主，对峰值压力和设施瓶颈的刻画仍不充分。对于AI来说，年度总量可能掩盖极端高温周的冷却补水、短时接入能力和污水处理超负荷。因此，本研究将日尺度作为主时间步，并以极端周小时序列检验时间聚合是否改变承载判断。

2.2　城市水—能—碳关系研究

城市水系统不是被动的水量输送网络，取水、加压、给水处理、污水处理、深度再生和跨区输配都需要能源。水源选择和处理深度的变化，可能在降低本地淡水取用的同时增加能耗和运营碳排放。因此，水—能—碳关系研究强调边界定义、过程能耗和生命周期尺度的一致性。

这一研究传统对AI课题有两点启示。第一，不能把“现场节水”直接等同于“总体环境改善”，尤其当再生水需要深度膜处理或远距离输送时，节水可能伴随能耗和碳排放转移。第二，本地人工水系统的物理水流必须与异地电力水足迹分开核算，否则会将城市水务部门能够治理的压力与供应链环境影响混为一谈。

2.3　AI与数据中心资源环境足迹研究

AI环境研究经历了从训练阶段能耗和碳排放，到模型部署、服务器制造、电子废物和基础设施水足迹的扩展。Green AI、机器学习碳足迹报告和数据中心工作负荷研究逐渐形成了PUE、WUE、利用率、服务器效率等指标体系。近期研究进一步采用情景分析和概率传播，强调AI水足迹存在显著的空间、时间和技术依赖。

这些研究的共同贡献是揭示AI水足迹受计算负荷、设施效率、冷却方式、气候与电力结构共同影响，因而不存在脱离条件的单位水耗常数[6–9,38,39]。研究边界仍多停留在服务器、设施或区域足迹：补水水源、排污去向、再生水竞争和城市设施余量通常没有在同一过程链中展开。近期关于公共供水容量和数据中心—污水厂协同的研究已经把问题推进到城市接口，但尚未形成兼顾水量、水质、时间尺度和瓶颈转换的统一模型[41,78,79]。

本研究沿用已有WUE工程关系并将其嵌入城市水系统，重点考察AI负荷进入城市网络后的边际响应和边界机制；WUE作为绩效指标，与完整水流闭合共同使用。

2.4　数据中心冷却与再生水工程研究

数据中心冷却技术可以从服务器侧热捕集和设施侧末端排热两个层次理解。空气冷却、direct-to-chip液冷和浸没式液冷改变热量从芯片传递到冷却介质的方式，但现场水耗最终还取决于末端采用蒸发湿式、干式、干湿混合或其他水侧排热方式。将液冷简单等同于“零水耗”会忽略末端散热和水质管理。

已有工程研究为腐蚀、结垢、生物生长、浓缩倍数和深度处理提供了参数与机理基础[48–52]；新近的数—水共生研究进一步表明，数据中心与污水厂之间存在空间和热量协同潜力[41]。但这些工作尚未在同一城市边界内同时刻画既有用户竞争、污水源量、输配能力和极端时段服务约束，这正是本研究的切入点。

2.5　水资源承载能力与可靠性研究

传统水资源承载力研究关注一定资源禀赋和生态约束下能够支撑多大的社会经济活动，常用总量平衡、系统动力学、情景分析和综合评价方法。可靠性—恢复性—脆弱性框架则把系统绩效从“是否有水”拓展到服务满足的频率、恢复速度和缺水损失。

AI承载容量与传统承载力既有联系又有差异。AI规模以MW/GW IT容量表征，水需求受技术架构和气候影响，且与居民基本服务存在竞争关系。因此，承载边界应同时考虑水源可用量、给水和污水设施、再生水生产和输配、峰值服务能力、局部接入和水质约束，并明确服务优先级与风险容忍。

2.6　文献评述与研究空白

综合上述研究，本课题的空白不是“缺少一种算法”，而是现有知识链条尚未闭合。设施足迹研究能够把计算量转换为水耗，城市水系统研究能够描述供水—污水—再生水循环，但两者之间缺少一个以AI IT容量为驱动、以城市人工水系统为边界、能够在日尺度上验证的过程接口。现有研究也缺少对“相同年度水量为何产生不同系统压力”的时间尺度解释，以及对水源、给水、污水和再生水瓶颈转换的机制识别。

本研究据此凝练三条科学矛盾：第一，设施补水的规模增长与城市水流结构变化并非同一件事；第二，相同年用水量可能对应不同的峰值压力和服务可靠性；第三，增加再生水或扩充设施改变的不只是容量大小，也会改变决定容量的主导环节。三条矛盾分别对应“发生了什么—为什么边界不同—边界由什么控制”的递进认识。

为使“代谢结构重构”具有可检验含义，本文把AI边际响应分为三个层次：第一层是规模响应，即各水流路径比例近似稳定、总量随容量同步变化；第二层是结构响应，即水源结构、Withdrawal—Consumption—Return Flow比例或污水—再生水内部循环发生持续偏移；第三层是约束状态转换，即主导约束从水源转向给水、从给水转向污水/再生水或从水量转向水质。只有同时观察到结构指标的系统性变化并排除单一容量截断造成的机械折点，才将结果解释为结构重构。

第3章　科学问题、研究目标与研究内容

3.1　核心科学问题

围绕研究空白，论文将“AI负荷如何改变城市人工水系统”作为总问题，并分解为三个相互递进的科学问题：

科学问题一（代谢响应）：AI算力基础设施通过怎样的取水—消耗—回流—再生利用链条影响城市人工水系统？当AI规模接近城市关键设施的剩余容量时，系统响应何时由水量增加转变为水源结构、循环路径和设施负荷的重构？

科学问题二（时间尺度边界）：AI负荷的连续性、冷却对高温的敏感性以及与传统城市需水峰值的耦合，如何造成年均、日尺度和极端小时尺度承载边界的差异？年均评价在何种城市状态下会高估可承载AI规模？

科学问题三（瓶颈转换）：随着AI容量、城市基础设施裕度和再生水配置变化，水源、给水、污水、再生水、局部接入和水质约束如何成为主导或共同主导瓶颈？不同干预措施的有效区间和边际收益转折由什么决定？

3.2　总体目标

建立AI—城市人工水系统代谢动态模型和条件化承载分析框架，形成从AI IT容量、热排散和冷却补水到城市给水—污水—再生水网络的可验证闭环；揭示AI扩张引起的代谢结构变化、时间尺度依赖边界和瓶颈迁移机制；在明确适用参数域和不确定性范围的前提下，为城市算力布局与水务设施协同规划提供可解释的容量依据。

3.3　研究假设

H1（代谢结构假设）：低AI负荷主要产生规模效应；当AI负荷接近关键设施剩余容量或改变水源配置时，淡水依赖、消耗—回流比例和再生水路径出现非线性结构变化。

H2（时间尺度假设）：AI连续负荷与高温冷却需求具有同步增强特征，年均容量边界通常高于日尺度边界，极端周小时边界进一步受峰值重合、湿球温度和干旱水源折减控制。

H3（瓶颈迁移假设）：AI扩张和干预措施改变的不仅是边界位置，也改变边界由谁决定；再生水扩容在源量、水质、处理、输配或既有用户竞争约束下存在边际收益转折。

3.4　研究内容与论文逻辑

四项研究内容并非四个并列的工程模块：第一项模型构建是统一实验平台，后三项分别承担现象识别、边界解释和干预分析，论文的科学贡献集中在代谢响应、时间尺度边界与瓶颈转换三类认识。

表3-1　研究内容、核心任务与预期产出

| 研究内容 | 核心任务 | 拟回答的问题 | 预期形成 |
| --- | --- | --- | --- |
| 一、耦合模型 | 构建AI节点、城市水循环、水量水质守恒和参数证据库 | AI如何进入城市水流网络 | 可验证AI-UWM模型 |
| 二、代谢响应 | 容量梯度、冷却/水源对照、AI与普通工业对照 | 何时由规模效应转为结构重构 | 代谢响应规律 |
| 三、承载边界 | 年均—日—小时、复合气候、CAWCC求解 | 平均与峰值边界为何不同 | 条件化边界和提前机制 |
| 四、瓶颈与干预 | 约束转换、再生水边际收益、负担转移 | 什么措施在什么条件下有效 | 瓶颈图谱和规划建议 |

3.5　研究边界与不纳入内容

本论文聚焦城市水系统的物理和工程机制，不将芯片级热仿真、完整管网瞬态水力、电网扩容优化、多目标容量优化和长期适应路径作为核心任务。水—能—碳—成本比较在核心模型通过验证后作为固定AI容量的应用检验，用于识别节水措施可能造成的负担转移，用于补充说明承载边界下的综合环境代价。

第4章　模型与研究方法

4.1　总体研究框架

模型采用“过程层—城市系统层—边界分析层”三层结构。过程层描述AI服务器负载、能耗、热排散、冷却补水、蒸发、漂水、排污和厂内回用；城市系统层连接外部水源、给水处理、配水、传统用户、污水处理、再生水处理、储存和最终排放；边界分析层在给定城市状态和情景下扫描AI容量，计算代谢指标、服务可靠性、CAWCC和主导瓶颈。三层之间通过水量、水质和时间序列接口连接，所有内部回用不重复计入外部输入。

4.2　城市人工水系统边界与守恒关系

设第t日进入城市人工水系统的外部水量为I_t，最终排向环境的水量为E_t，不可恢复消耗为C_t，显式储量变化为ΔS_t，则顶层守恒关系为I_t=E_t+C_t+ΔS_t。原水经给水处理和配水后进入传统用户与AI节点；用户用水按照消耗和回流分解，AI排污按照厂内回用、处理损失、污水回流和外排分流。给水、污水和再生水各自设置处理能力、输配能力、储量和既有需求，漏损、溢流和未满足需求进入独立账户。

4.3　AI设施过程模型

AI IT容量是主要规模变量。实际IT负荷由容量和负载率共同决定，设施总能耗由IT能耗与PUE关系得到，纳入边界的电能大部分转化为需要排散的热量。服务器侧热捕集采用空气、direct-to-chip液冷和浸没式液冷等架构分类；设施侧末端排热采用干式、湿式和混合方式分类。湿侧热负荷通过显热与潜热分解得到蒸发量，循环水根据浓缩倍数和水质约束计算排污量，补水量由蒸发、漂水和排污共同决定。

冷却排污并不自动等于城市污水回流。模型首先判断厂内回收比例和处理损失，再将剩余部分连接到城市污水系统；若排污水的TDS、温度或药剂负荷超过污水厂或再生水进水要求，则增加独立水质约束。这样可以区分可回用排污、处理损失和城市污水回流，并分析水量充足而水质不适用时再生水系统的约束。

4.4　水源、再生水与分配机制

AI目标再生水份额只是配置意图，实际可供量受污水源量、深度处理能力、再生水储量、输配能力和既有用户需求共同决定。满足居民基本服务和关键公共服务后，剩余可供水量按情景规则分配给其他用户与AI。模型分别记录目标再生水份额、实际再生水供应、淡水补足量和未满足需求，防止用“目标比例”替代真实供水。

4.5　代谢指标与对照设计

研究同时报告绝对量和归一化量。绝对量包括AI边际取水、消耗、回流、污水负荷和再生水供给；归一化指标包括AI取水/城市基准供水、AI峰值需求/接入能力、淡水依赖率、内部循环率、消耗率和回流率。对互斥且共同分母的路径计算结构距离，用于区分整体规模放大和水流结构重构。设置等年用水量、等峰值热负荷和等接入能力的普通工业冷却节点对照，仅改变负荷连续性、峰值时序和气候响应，据此区分一般高密度工业效应与AI负荷的特异性贡献。

4.6　CAWCC求解方法

CAWCC定义为给定条件下满足全部核心约束的最大AI IT容量。约束集合包括外部水源压力、给水处理和局部接入、污水潜在负荷和溢流、再生水服务、水质可接纳能力、峰值服务能力以及居民基本服务。求解先进行容量网格扫描，检查可行域是否单调；单调时在边界附近细化，非单调时报告全部可行区间、首次失效容量和最终失效容量。边界结果同时列出城市状态、技术架构、水源政策、气候情景、服务优先级和风险容忍条件。

4.7　时间尺度、可靠性与复合压力

年均分析用于获得长期规模关系，日尺度分析用于识别季节和需求峰值，小时校核用于检验极端周内的短时失稳。高温、干旱和传统需求峰值分别构造单因素与组合压力，保持它们的时间相关结构，保持边际变量的时间相关结构，而非将极值简单相乘。可靠性以基本服务满足的时间比例表示，恢复性关注失效后的恢复速度，脆弱性以缺水量和持续时间刻画。年均边界与峰值边界之差用于量化时间聚合造成的边界提前。

4.8　不确定性与敏感性分析

不确定性分析围绕三个问题展开：哪些参数真正影响容量边界，参数关系如何改变归因，以及不同未来情景下结论是否保持方向一致。负载率、PUE、CoC、污水回流率、再生水回收率和水质阈值采用条件或联合分布传播；AI增长、气候路径、政策和扩容时点采用情景集合。先以Morris筛选重要变量，再对近似独立输入实施Sobol分解，对具有统计相关性的输入使用Shapley effects，从而使算法选择服务于科学问题，而不是形成方法清单。

第5章　前期基础与可行性

5.1　前期研究基础

前期已完成城市资源代谢模拟框架的主体结构，形成水源、给水、终端需求、污水处理和再生利用的日尺度计算模块，并建立了情景切换与不确定性分析接口。现有原型已经能够按节点记录外部输入、内部回用、最终排放和储量变化；下一步是在这一可运行骨架上接入AI冷却节点，从而把研究重点放在AI节点引入后的科学响应。

现有研究积累形成了三项可直接用于论文工作的基础：一是已将城市水系统表示为节点—路径网络，并区分外部输入、内部回用和最终排放；二是已把服务器侧热捕集与设施侧末端热排散分开，能够在同一框架中比较空气、液冷、干式和湿式方案；三是已建立参数证据分级表，分别记录公开统计、工程文献、企业披露和情景假设，为后续校准和敏感性分析提供可追溯入口。

5.2　数据可得性与案例策略

表5-1　分层数据方案与证据用途

| 数据层级 | 数据内容 | 获取与使用方式 | 对结论的作用 |
| --- | --- | --- | --- |
| 深度案例 | 连续日尺度供水、污水、再生水、气象及设施能力 | 公开统计、年报、管理部门资料；时间留出校准与验证 | 约束真实城市响应与峰值边界 |
| 外部锚点 | 水资源禀赋、热环境、供排水能力、再生水利用和典型运行指标 | 公开资料和可比统计；不参与拟合 | 检验参数范围、方向和瓶颈类型 |
| 工程与文献 | 冷却架构、PUE、CoC、蒸发排污、水质和处理能耗 | 标准、工程文献、公开Benchmark | 形成条件分布和结构对照 |
| 敏感数据 | 企业逐小时负载、用水、水质和局部接入 | 若可获得则用于校核；否则使用区间与替代剖面 | 不作为模型成立的单一前提 |

深度案例从北京、天津、贵阳等候选城市中按四项标准筛选：

5.3　模型验证与证据等级

模型验证采用“代码验证—过程验证—独立验证—外部核验—结构稳健性”递进方式。代码验证检查质量守恒、非负性、容量限制、储量更新和随机复现；过程验证将IT能耗、PUE、蒸发、排污、再生水处理与公开工程关系比较；独立验证使用时间留出数据；外部核验比较锚点城市的响应方向和瓶颈类型；结构稳健性比较机理冷却模型与经验WUE模型。不同证据等级在结果中分开标注，文献数量级一致不替代真实时间验证。

5.4　可行性分析

理论可行性：城市代谢、综合水系统、数据中心冷却和可靠性评价已有成熟理论接口，本研究的增量在于耦合和机制解释。

数据可行性：核心城市水务和气象数据可由公开统计与管理资料支撑；商业敏感数据采取工程Benchmark、区间参数和条件剖面替代。

计算可行性：模型按模块开发，优先在边界附近计算；高维敏感性分析只对证据充分的变量实施。

论文可行性：模型构建、代谢响应、承载边界、瓶颈与干预四部分具有相对独立的研究问题，可形成连续论文产出。

5.5　关键难点与应对措施

表5-2　关键难点、研究影响与解决路径

| 难点 | 可能后果 | 应对措施 |
| --- | --- | --- |
| AI运行数据不足 | 难以校准逐小时负荷和水耗 | 采用公开工程参数、文献Benchmark和保留日总量的条件剖面；把不可辨识性单独报告 |
| 水量与水质耦合复杂 | 水量余量掩盖水质失效 | 追踪TDS、温度和药剂负荷，设置水质独立约束 |
| 反馈导致可行域非单调 | 只报告最大值会掩盖中间失效 | 先网格扫描，报告全部可行区间和首次/最终失效 |
| 城市间可比性不足 | 跨城市结论被过度外推 | 使用无量纲裕度和适用参数域，外部锚点只承担方向核验 |

第6章　研究计划、创新与预期成果

6.1　拟开展的关键研究

研究一：AI—城市水系统耦合机制。明确AI设施在城市水网络中的节点位置、边界和水量水质路径，构建最小闭环并完成验证。该研究解决“AI负荷如何转化为Withdrawal、Consumption和Return Flow”的基础问题，为后续所有边界分析提供统一分母和守恒基础。

研究二：AI扩张下城市水代谢响应。沿AI容量梯度改变负荷率、冷却架构和水源配置，比较AI情景与Baseline，并设置等效普通工业冷却对照。研究重点由平均耗水系数转向识别在何种AI容量比、设施裕度和水源组合下，淡水依赖、内部循环和设施负荷出现结构性转折。

研究三：时间尺度依赖的CAWCC。对同一城市状态分别计算年均、日尺度和极端周小时边界，逐步加入高温、干旱和传统需求峰值。通过峰值重合指数、湿球温度、水源折减和设施剩余裕度解释边界提前，回答年均评价何时系统性高估AI承载能力。

研究四：瓶颈转换与干预边界。对水源、给水、污水、再生水、接入和水质约束分别记录首次达到失效条件的AI容量，绘制城市状态—技术—瓶颈转换图。进一步增加再生水处理/输配、给水和污水能力，比较单位干预带来的CAWCC提升，识别源量受限、质量受限、处理受限、输配受限和竞争受限的边际转折。

6.2　技术路线

技术路线分为六步：第一步完成文献和数据审计，确定案例边界与参数证据；第二步将AI设施过程嵌入城市人工水系统并完成水量水质守恒；第三步利用深度案例进行校准和时间留出验证，并用外部锚点核验范围和方向；第四步开展容量梯度、时间尺度和复合压力实验；第五步求解CAWCC并识别瓶颈转换；第六步进行不确定性、结构稳健性和干预边际收益分析。每一步的输出都是下一步的输入，待闭环验证后再开展边界分析与扩展应用。

6.3　研究任务的优先级与工作量控制

博士论文以四项任务形成闭合主线。第一优先级是AI-UWM物理闭合、代谢响应、时间尺度CAWCC和瓶颈转换；第二优先级是再生水边际收益与参数不确定性；在前两级形成稳定结果后，再拓展完整水质过程、水—能—碳比较和跨城市泛化。这样的安排使每一阶段都能产出科学结果，同时保留技术扩展的空间。

6.4　预期创新点

创新一（研究尺度）：把AI算力中心从设施水足迹研究中的“黑箱”转化为城市人工水系统中的可计算节点，建立从IT容量到给水—污水—再生水的水量水质闭合。

创新二（科学机制）：提出并检验“连续AI负荷—气候敏感冷却—城市峰值需求”共同形成时间尺度依赖承载边界的机制，解释相同年用水量为何产生不同峰值风险。

创新三（承载理论）：将AI承载容量定义为由水源、设施、峰值、接入和水质共同决定的条件化可行边界，揭示AI扩张和干预过程中主导瓶颈迁移及再生水边际收益转折。

6.5　工作计划

表6-1　研究阶段、主要工作与阶段产出

| 阶段 | 时间 | 主要工作 | 阶段产出 |
| --- | --- | --- | --- |
| 第一阶段 | 2026.09—2027.06 | 数据审计、案例筛选、文献综述、最小闭环、单元测试 | 研究数据库、AI-UWM原型、方法论文初稿 |
| 第二阶段 | 2027.07—2028.03 | 深度案例校准、时间留出验证、外部锚点和对照实验 | 代谢响应结果、论文1、可复现代码版本 |
| 第三阶段 | 2028.04—2028.12 | 年均—日—小时比较、高温干旱复合压力、CAWCC | 时间尺度边界结果、论文2 |
| 第四阶段 | 2029.01—2029.09 | 不确定性、瓶颈转换、再生水和设施干预 | 边界图谱、机制论文、参数证据库 |
| 第五阶段 | 2029.10—2030.06 | 固定容量水—能—碳比较、论文整合、预答辩与送审 | 博士论文、代码和数据治理文档 |

6.6　预期科学认识

明确AI负荷相对于城市基准水流达到何种相对规模后，系统响应由总量增加转为代谢结构重构。

识别年均、日尺度和极端时段容量边界差异的控制因素，说明峰值重合和设施裕度在不同城市中的作用。

建立水源—给水—污水—再生水—接入约束的瓶颈转换条件，区分稳健主导区与竞争约束区。

解释再生水扩容何时有效、何时受源量或水质等条件限制，并识别节水措施可能造成的能耗、碳排放或成本转移。

6.7　预期成果

形成经过代码验证、过程Benchmark、独立时间验证和外部锚定的AI-UWM模型及版本化代码。

形成AI容量—代谢结构—分系统峰值压力响应面和适用参数域。

形成工程安全与风险条件下的CAWCC边界、边界提前量和瓶颈转换图谱。

发表围绕模型、时间尺度边界和瓶颈/干预机制的系列论文，提交数据字典、参数证据表和可重复运行脚本。

6.8　成果解释与推广边界

研究结论将分为条件规律与跨城市机制方向两类：前者对应具体城市状态、技术架构和气候情景，后者要求在外部锚点和结构对照中保持方向一致。参数以证据等级和区间表达，城市本地物理水流与异地电力水足迹分账户呈现，AI节点与等年用水量普通工业节点并列比较。规划建议以容量区间、适用域和主导瓶颈呈现，用于支持方案比较和前期论证。

参考文献

[1] International Energy Agency. Key Questions on Energy and AI[R]. Paris: IEA, 2026.

[2] International Energy Agency. Energy and AI[R]. Paris: IEA, 2025.

[3] Shehabi A, Smith S J, Hubbard A, Newkirk A, Lei N, Siddik M A B, et al. 2024 United States Data Center Energy Usage Report[R]. Berkeley: Lawrence Berkeley National Laboratory, 2024. DOI: 10.71468/P1WC7Q.

[4] Masanet E, Shehabi A, Lei N, Smith S, Koomey J. Recalibrating global data center energy-use estimates[J]. Science, 2020, 367(6481): 984-986. DOI: 10.1126/science.aba3758.

[5] de Vries A. The growing energy footprint of artificial intelligence[J]. Joule, 2023, 7(10): 2191-2194. DOI: 10.1016/j.joule.2023.09.004.

[6] Mytton D. Data centre water consumption[J]. npj Clean Water, 2021, 4: 11. DOI: 10.1038/s41545-021-00101-w.

[7] Lei N, Lu J, Shehabi A, Masanet E R. The water use of data center workloads: A review and assessment of key determinants[J]. Resources, Conservation and Recycling, 2025, 219: 108310. DOI: 10.1016/j.resconrec.2025.108310.

[8] Xiao T, Fuso Nerini F, Matthews H D, Tavoni M, You F. Environmental impact and net-zero pathways for sustainable artificial intelligence servers in the USA[J]. Nature Sustainability, 2025, 8: 1541-1553. DOI: 10.1038/s41893-025-01681-y.

[9] Jiang F, Duan C, Chen B. Facility-level energy-driven water footprint and scarcity implications of Chinese data centers: A bottom-up analysis and scenario-based projection[J]. Applied Energy, 2025, 399: 126522. DOI: 10.1016/j.apenergy.2025.126522.

[10] 国家发展改革委, 工业和信息化部, 国家能源局, 国家数据局. 数据中心绿色低碳发展专项行动计划[Z]. 发改环资〔2024〕970号, 2024.

[11] Wolman A. The metabolism of cities[J]. Scientific American, 1965, 213(3): 178-190. DOI: 10.1038/scientificamerican0965-178.

[12] Kennedy C, Cuddihy J, Engel-Yan J. The changing metabolism of cities[J]. Journal of Industrial Ecology, 2007, 11(2): 43-59. DOI: 10.1162/jie.2007.1107.

[13] Kennedy C, Pincetl S, Bunje P. The study of urban metabolism and its applications to urban planning and design[J]. Environmental Pollution, 2011, 159(8-9): 1965-1973. DOI: 10.1016/j.envpol.2010.10.022.

[14] Castán Broto V, Allen A, Rapoport E. Interdisciplinary perspectives on urban metabolism[J]. Journal of Industrial Ecology, 2012, 16(6): 851-861. DOI: 10.1111/j.1530-9290.2012.00556.x.

[15] Behzadian K, Kapelan Z, Venkatesh G, Brattebø H, Sægrov S. WaterMet²: a tool for integrated analysis of sustainability-based performance of urban water systems[J]. Drinking Water Engineering and Science, 2014, 7: 63-72. DOI: 10.5194/dwes-7-63-2014.

[16] Behzadian K, Kapelan Z. Modelling metabolism based performance of an urban water system using WaterMet²[J]. Resources, Conservation and Recycling, 2015, 99: 84-99. DOI: 10.1016/j.resconrec.2015.03.015.

[17] Urich C, Rauch W. Modelling the urban water cycle as an integrated part of the city: a review[J]. Water Science and Technology, 2014, 70(11): 1857-1872. DOI: 10.2166/wst.2014.363.

[18] Bach P M, Rauch W, Mikkelsen P S, McCarthy D T, Deletic A. A critical review of integrated urban water modelling—Urban drainage and beyond[J]. Environmental Modelling & Software, 2014, 54: 88-107. DOI: 10.1016/j.envsoft.2013.12.018.

[19] Dobson B, Liu L, Mijic A. Modelling water quantity and quality for integrated water cycle management with the Water Systems Integrated Modelling framework (WSIMOD) software[J]. Geoscientific Model Development, 2024, 17: 4495-4513. DOI: 10.5194/gmd-17-4495-2024.

[20] Cui D, Zeng W, Ma B, Zhuo Y, Xie Y. Ecological network analysis of an urban water metabolic system: Integrated metabolic processes of physical and virtual water[J]. Science of the Total Environment, 2021, 787: 147432. DOI: 10.1016/j.scitotenv.2021.147432.

[21] Nezami N, Zarghami M, Tizghadam M, Abbasi M. A novel hybrid systemic modeling into sustainable dynamic urban water metabolism management: Case study[J]. Sustainable Cities and Society, 2022, 85: 104065. DOI: 10.1016/j.scs.2022.104065.

[22] Emamjomehzadeh O, Kerachian R, Emami-Skardi M J, Momeni M. Combining urban metabolism and reinforcement learning concepts for sustainable water resources management: A nexus approach[J]. Journal of Environmental Management, 2023, 329: 117046. DOI: 10.1016/j.jenvman.2022.117046.

[23] Fan J L, Kong L S, Wang H, Zhang X. A water-energy nexus review from the perspective of urban metabolism[J]. Ecological Modelling, 2019, 392: 128-136. DOI: 10.1016/j.ecolmodel.2018.11.019.

[24] Kenway S J, Lant P A, Priestley A, Daniels P. The connection between water and energy in cities: a review[J]. Water Science and Technology, 2011, 63(9): 1983-1990. DOI: 10.2166/wst.2011.070.

[25] Kenway S J, Binks A, Lane J, Lant P A, Lam K L, Simms A. A systemic framework and analysis of urban water energy[J]. Environmental Modelling & Software, 2015, 73: 272-285. DOI: 10.1016/j.envsoft.2015.08.009.

[26] Nair S, George B, Malano H M, Arora M, Nawarathna B. Water–energy–greenhouse gas nexus of urban water systems: Review of concepts, state-of-art and methods[J]. Resources, Conservation and Recycling, 2014, 89: 1-10. DOI: 10.1016/j.resconrec.2014.05.007.

[27] Lee M, Keller A A, Chiang P C, Den W, Wang H, Hou C H, et al. Water-energy nexus for urban water systems: A comparative review on energy intensity and environmental impacts in relation to global water risks[J]. Applied Energy, 2017, 205: 589-601. DOI: 10.1016/j.apenergy.2017.08.002.

[28] Stokes J R, Horvath A. Energy and air emission effects of water supply[J]. Environmental Science & Technology, 2009, 43(8): 2680-2687. DOI: 10.1021/es801802h.

[29] House-Peters L A, Chang H. Urban water demand modeling: Review of concepts, methods, and organizing principles[J]. Water Resources Research, 2011, 47(5): W05401. DOI: 10.1029/2010WR009624.

[30] Strubell E, Ganesh A, McCallum A. Energy and policy considerations for deep learning in NLP[C]//Proceedings of the 57th Annual Meeting of the Association for Computational Linguistics. Florence: ACL, 2019: 3645-3650. DOI: 10.18653/v1/P19-1355.

[31] Henderson P, Hu J, Romoff J, Brunskill E, Jurafsky D, Pineau J. Towards the systematic reporting of the energy and carbon footprints of machine learning[J]. Journal of Machine Learning Research, 2020, 21(248): 1-43.

[32] Lannelongue L, Grealey J, Inouye M. Green Algorithms: Quantifying the carbon footprint of computation[J]. Advanced Science, 2021, 8(12): 2100707. DOI: 10.1002/advs.202100707.

[33] Schwartz R, Dodge J, Smith N A, Etzioni O. Green AI[J]. Communications of the ACM, 2020, 63(12): 54-63. DOI: 10.1145/3381831.

[34] Patterson D, Gonzalez J, Hölzle U, Le Q, Liang C, Munguia L M, et al. The carbon footprint of machine learning training will plateau, then shrink[J]. Computer, 2022, 55(7): 18-28. DOI: 10.1109/MC.2022.3148714.

[35] Luccioni A S, Viguier S, Ligozat A L. Estimating the carbon footprint of BLOOM, a 176B parameter language model[J]. Journal of Machine Learning Research, 2023, 24(253): 1-15.

[36] Luccioni A S, Jernite Y, Strubell E. Power hungry processing: Watts driving the cost of AI deployment?[C]//Proceedings of the 2024 ACM Conference on Fairness, Accountability, and Transparency. New York: ACM, 2024: 85-99. DOI: 10.1145/3630106.3658542.

[37] Wang P, Zhang L Y, Tzachor A, Chen W Q. E-waste challenges of generative artificial intelligence[J]. Nature Computational Science, 2024, 4: 818-823. DOI: 10.1038/s43588-024-00712-6.

[38] Li P, Yang J, Islam M A, Ren S. Making AI Less “Thirsty”: Uncovering and addressing the secret water footprint of AI models[J]. Communications of the ACM, 2025, 68(7): 54-61. DOI: 10.1145/3724499.

[39] Herrera M, Xie X, Menapace A, Zanfei A, Brentan B M. Sustainable AI infrastructure: A scenario-based forecast of water footprint under uncertainty[J]. Journal of Cleaner Production, 2025, 526: 146528. DOI: 10.1016/j.jclepro.2025.146528.

[40] Alissa H, Nick T, Raniwala A, et al. Using life cycle assessment to drive innovation for sustainable cool clouds[J]. Nature, 2025, 641: 331-338. DOI: 10.1038/s41586-025-08832-3.

[41] Wang A, Zhang C, Xu T, Savić D, Jiang J, Xiao P, et al. Global data–water symbiosis reduces AI infrastructure’s carbon and water footprint[J]. Environmental Science and Ecotechnology, 2026, 31: 100702. DOI: 10.1016/j.ese.2026.100702.

[42] Ebrahimi K, Jones G F, Fleischer A S. A review of data center cooling technology, operating conditions and the corresponding low-grade waste heat recovery opportunities[J]. Renewable and Sustainable Energy Reviews, 2014, 31: 622-638. DOI: 10.1016/j.rser.2013.12.007.

[43] Zhang H, Shao S, Xu H, Zou H, Tian C. Free cooling of data centers: A review[J]. Renewable and Sustainable Energy Reviews, 2014, 35: 171-182. DOI: 10.1016/j.rser.2014.04.017.

[44] Daraghmeh H M, Wang C C. A review of current status of free cooling in datacenters[J]. Applied Thermal Engineering, 2017, 114: 1224-1239. DOI: 10.1016/j.applthermaleng.2016.10.093.

[45] Yuan X, Zhou X, Pan Y, Kosonen R, Cai H, Gao Y, Wang Y. Phase change cooling in data centers: A review[J]. Energy and Buildings, 2021, 236: 110764. DOI: 10.1016/j.enbuild.2021.110764.

[46] Zhang Y, Zhao Y, Dai S, Nie B, Ma H, Li J, et al. Cooling technologies for data centres and telecommunication base stations—A comprehensive review[J]. Journal of Cleaner Production, 2022, 334: 130280. DOI: 10.1016/j.jclepro.2021.130280.

[47] Chen H, Li D. Current status and challenges for liquid-cooled data centers[J]. Frontiers in Energy Research, 2022, 10: 952680. DOI: 10.3389/fenrg.2022.952680.

[48] Hsieh M K, Li H, Chien S H, Monnell J D, Chowdhury I, Dzombak D A, Vidic R D. Corrosion control when using secondary treated municipal wastewater as alternative makeup water for cooling tower systems[J]. Water Environment Research, 2010, 82(12): 2346-2356. DOI: 10.2175/106143010X12681059117094.

[49] Liu W, Chien S H, Dzombak D A, Vidic R D. Mineral scaling mitigation in cooling systems using tertiary-treated municipal wastewater[J]. Water Research, 2012, 46(14): 4488-4498. DOI: 10.1016/j.watres.2012.05.041.

[50] Chien S H, Chowdhury I, Hsieh M K, Li H, Dzombak D A, Vidic R D. Control of biological growth in recirculating cooling systems using treated secondary effluent as makeup water with monochloramine[J]. Water Research, 2012, 46(19): 6508-6518. DOI: 10.1016/j.watres.2012.09.027.

[51] Choudhury M R, Hsieh M K, Vidic R D, Dzombak D A. Corrosion management in power plant cooling systems using tertiary-treated municipal wastewater as makeup water[J]. Corrosion Science, 2012, 61: 231-241. DOI: 10.1016/j.corsci.2012.04.042.

[52] Kornboonraksa T. Using tertiary-treated municipal wastewater as makeup water by reverse osmosis membrane[J]. Desalination and Water Treatment, 2016, 57(16): 7422-7431. DOI: 10.1080/19443994.2015.1024937.

[53] 段春青, 刘昌明, 陈晓楠, 柳文华, 郑红星. 区域水资源承载力概念及研究方法的探讨[J]. 地理学报, 2010, 65(1): 82-90. DOI: 10.11821/xb201001009.

[54] Qin G, Li H, Wang X, Ding J. Research on water resources design carrying capacity[J]. Water, 2016, 8(4): 157. DOI: 10.3390/w8040157.

[55] Feng L H, Zhang X C, Luo G Y. Application of system dynamics in analyzing the carrying capacity of water resources in Yiwu City, China[J]. Mathematics and Computers in Simulation, 2008, 79(3): 269-278. DOI: 10.1016/j.matcom.2007.11.018.

[56] Wang X J, Zhang J Y, Liu J F, Wang G Q, He R M, Elmahdi A, Elsawah S. Water resources planning and management based on system dynamics: A case study of Yulin city[J]. Environment, Development and Sustainability, 2011, 13(2): 331-351. DOI: 10.1007/s10668-010-9264-6.

[57] Wang G, Xiao C, Qi Z, Meng F, Liang X. Development tendency analysis for the water resource carrying capacity based on system dynamics model and the improved fuzzy comprehensive evaluation method in Changchun City, China[J]. Ecological Indicators, 2021, 122: 107232. DOI: 10.1016/j.ecolind.2020.107232.

[58] Wang X, Liu L, Zhang S, Gao C. Dynamic simulation and comprehensive evaluation of the water resources carrying capacity in Guangzhou City, China[J]. Ecological Indicators, 2022, 135: 108528. DOI: 10.1016/j.ecolind.2021.108528.

[59] Jia Z, Cai Y, Chen Y, Zeng W. Regionalization of water environmental carrying capacity for supporting the sustainable water resources management and development in China[J]. Resources, Conservation and Recycling, 2018, 134: 282-293. DOI: 10.1016/j.resconrec.2018.03.030.

[60] Zhou X Y, Zheng B, Khu S T. Validation of the hypothesis on carrying capacity limits using the water environment carrying capacity[J]. Science of the Total Environment, 2019, 665: 774-784. DOI: 10.1016/j.scitotenv.2019.02.146.

[61] He C, Liu Z, Wu J, Pan X, Fang Z, Li J, Bryan B A. Future global urban water scarcity and potential solutions[J]. Nature Communications, 2021, 12: 4667. DOI: 10.1038/s41467-021-25026-3.

[62] Hashimoto T, Stedinger J R, Loucks D P. Reliability, resiliency, and vulnerability criteria for water resource system performance evaluation[J]. Water Resources Research, 1982, 18(1): 14-20. DOI: 10.1029/WR018i001p00014.

[63] Gupta H V, Kling H, Yilmaz K K, Martinez G F. Decomposition of the mean squared error and NSE performance criteria: Implications for improving hydrological modelling[J]. Journal of Hydrology, 2009, 377(1-2): 80-91. DOI: 10.1016/j.jhydrol.2009.08.003.

[64] Morris M D. Factorial sampling plans for preliminary computational experiments[J]. Technometrics, 1991, 33(2): 161-174. DOI: 10.1080/00401706.1991.10484804.

[65] Campolongo F, Cariboni J, Saltelli A. An effective screening design for sensitivity analysis of large models[J]. Environmental Modelling & Software, 2007, 22(10): 1509-1518. DOI: 10.1016/j.envsoft.2006.10.004.

[66] Sobol I M. Global sensitivity indices for nonlinear mathematical models and their Monte Carlo estimates[J]. Mathematics and Computers in Simulation, 2001, 55(1-3): 271-280. DOI: 10.1016/S0378-4754(00)00270-6.

[67] Saltelli A, Ratto M, Andres T, Campolongo F, Cariboni J, Gatelli D, et al. Global Sensitivity Analysis: The Primer[M]. Chichester: John Wiley & Sons, 2008. DOI: 10.1002/9780470725184.

[68] Razavi S, Gupta H V. What do we mean by sensitivity analysis? The need for comprehensive characterization of “global” sensitivity in Earth and environmental systems models[J]. Water Resources Research, 2015, 51(5): 3070-3092. DOI: 10.1002/2014WR016527.

[69] Mortazavi-Naeini M, Kuczera G, Kiem A S, Cui L, Henley B, Berghout B, Turner E. Robust optimization to secure urban bulk water supply against extreme drought and uncertain climate change[J]. Environmental Modelling & Software, 2015, 69: 437-451. DOI: 10.1016/j.envsoft.2015.02.021.

[70] Lempert R J, Groves D G. Identifying and evaluating robust adaptive policy responses to climate change for water management agencies in the American West[J]. Technological Forecasting and Social Change, 2010, 77(6): 960-974. DOI: 10.1016/j.techfore.2010.04.007.

[71] Haasnoot M, Kwakkel J H, Walker W E, ter Maat J. Dynamic adaptive policy pathways: A method for crafting robust decisions for a deeply uncertain world[J]. Global Environmental Change, 2013, 23(2): 485-498. DOI: 10.1016/j.gloenvcha.2012.12.006.

[72] Kwakkel J H, Haasnoot M, Walker W E. Developing dynamic adaptive policy pathways: A computer-assisted approach for developing adaptive strategies for a deeply uncertain world[J]. Climatic Change, 2015, 132(3): 373-386. DOI: 10.1007/s10584-014-1210-4.

[73] Sadr S M K, Casal-Campos A, Fu G, Farmani R, Ward S, Butler D. Strategic planning of the integrated urban wastewater system using adaptation pathways[J]. Water Research, 2020, 182: 116013. DOI: 10.1016/j.watres.2020.116013.

[74] Li R, Zhang D, Dong X, Gong D, Mo L. How can urban wastewater and reclaimed water systems tackle an uncertain future without regret?—Solutions based on adaptive pathway planning[J]. Journal of Environmental Management, 2026, 410: 130062. DOI: 10.1016/j.jenvman.2026.130062.

[75] Deb K, Pratap A, Agarwal S, Meyarivan T. A fast and elitist multiobjective genetic algorithm: NSGA-II[J]. IEEE Transactions on Evolutionary Computation, 2002, 6(2): 182-197. DOI: 10.1109/4235.996017.

[76] Eyring V, Bony S, Meehl G A, Senior C A, Stevens B, Stouffer R J, Taylor K E. Overview of the Coupled Model Intercomparison Project Phase 6 (CMIP6) experimental design and organization[J]. Geoscientific Model Development, 2016, 9: 1937-1958. DOI: 10.5194/gmd-9-1937-2016.

[77] Zscheischler J, Westra S, van den Hurk B J J M, Seneviratne S I, Ward P J, Pitman A, et al. Future climate risk from compound events[J]. Nature Climate Change, 2018, 8: 469-477. DOI: 10.1038/s41558-018-0156-3.

[78] Han Y, Li P, Wierman A, Ren S. Small Bottle, Big Pipe: Quantifying and Addressing the Impact of Data Centers on Public Water Systems[EB/OL]. arXiv:2603.02705, 2026. DOI: 10.48550/arXiv.2603.02705. （预印本）

[79] Akinade B A, Amanambu A C, Frame J M, Ren S. AI Data Centers and the Water Use Feedback Loop[EB/OL]. arXiv:2606.21760, 2026. DOI: 10.48550/arXiv.2606.21760. （预印本）

[80] IPCC. Climate Change 2021: The Physical Science Basis. Contribution of Working Group I to the Sixth Assessment Report of the Intergovernmental Panel on Climate Change[M]. Cambridge: Cambridge University Press, 2021. DOI: 10.1017/9781009157896.

[81] Song E, Nelson B L, Staum J. Shapley Effects for Global Sensitivity Analysis: Theory and Computation[J]. SIAM/ASA Journal on Uncertainty Quantification, 2016, 4(1): 1060-1083. DOI: 10.1137/15M1048070.

[82] 国家市场监督管理总局, 国家标准化管理委员会. 数据中心 资源利用 第6部分：水资源使用效率: GB/T 32910.6-2025[S]. 2025.

[83] 中华人民共和国住房和城乡建设部. 室外给水设计标准: GB 50013-2018[S]. 北京: 中国计划出版社, 2018.

[84] 中华人民共和国住房和城乡建设部. 室外排水设计标准: GB 50014-2021[S]. 北京: 中国计划出版社, 2021.

[85] 北京市水务局. 北京市水资源公报（2024年度）[R]. 2025. https://swj.beijing.gov.cn/zwgk/szygb/

[86] 天津水文水资源. 2024年天津市水资源公报[R]. 2025. https://www.tjwater.net.cn/nd.jsp?fromColId=2&id=1063

[87] 贵州省水利厅. 2024年贵州省水资源公报[R]. 2026. https://gzswj.guizhou.gov.cn/swyw/szy/

[88] 北京市水务局. 2024年北京市水务工作报告[R]. 2024. https://swj.beijing.gov.cn/zwgk/ghjhzj/202405/t20240521_3690638.html

[89] 北京市水务局. 本市去年用量达12.77亿立方米：再生水已成稳定的“第二水源”[EB/OL]. 2024. https://swj.beijing.gov.cn/swdt/swyw/202403/t20240325_3600506.html